SMART WATER MANAGEMENT FOR
URBAN GREEN INFRASTRUCTURE

A Low-Cost, Network-Resilient Edge-IoT Irrigation Node
with Deep-Learning Soil Analysis

FINAL PROJECT REPORT

Course: Design of Smart Cities

# Table of Contents

Table of Contents	2

Abstract	4

1. Introduction	4

1.1 Purpose	4

1.2 Scope	4

1.3 Smart-city context	5

1.4 Definitions, acronyms and abbreviations	5

1.5 Overview of this document	5

2. Software Requirements Specification (SRS)	6

2.1 Product perspective	6

2.2 Product functions	6

2.3 User characteristics and stakeholders	6

2.4 Functional requirements	6

2.5 Non-functional requirements	7

2.6 External interface requirements	7

2.7 Constraints, assumptions and dependencies	8

3. Software Design Specification (SDS)	8

3.1 Architectural design	8

3.2 Deployment design	9

3.3 Hardware design and component selection	9

3.4 Circuit design	10

3.5 Data design	11

3.6 Component and module design	12

3.7 Control-law design	12

3.8 Machine-learning pipeline design	13

3.9 Design decisions and rationale	13

4. Implementation	13

4.1 Firmware	13

4.2 Calibration procedure	14

4.3 Host telemetry collector	14

4.4 Classifier pipeline	14

4.5 Dashboard and advisory	14

4.6 Prototype assembly	14

4.7 Technology stack	15

5. Testing	15

5.1 Test strategy	15

5.2 Functional and fault-injection testing	15

5.3 Control-law validation	16

5.4 Classifier evaluation	17

5.5 Telemetry reliability	18

6. Outcome	18

6.1 Requirements traceability	18

6.2 Quantitative outcomes	19

6.3 Unique outcome: measuring evaluation inflation in soil-image classification	19

6.4 Outcome against the smart-city objective	20

6.5 Limitations	20

6.6 Future scope	20

6.7 Conclusion	21

References	21

# Abstract

Cities irrigate a great deal of land. Parks, urban forests, road medians, green belts, traffic islands, institutional lawns, green roofs and community gardens all need water, and in most municipalities that water is delivered on a fixed timer. A timer cannot tell that it rained yesterday, that one bed drains faster than another, or that a valve has failed. The result is simultaneous over-watering and under-watering across the same green space, in cities that are often already under water stress.

This project specifies, designs, implements and tests a low-cost irrigation node for exactly that setting. Built around an ESP32-WROOM-32, it measures soil moisture with a capacitive probe, measures air temperature and humidity with a DHT11, and switches a pump through a relay driven by a BC547B transistor. A two-threshold hysteresis rule — water below 30 % moisture, stop above 60 % — replaces the timer, and the complete decision loop executes on the microcontroller once per second.

The architectural decision that matters most in a smart-city context is where that decision is taken. The node does not ask a server whether to irrigate. A host computer logs its telemetry, classifies soil photographs and produces weather-aware watering advice, but is physically incapable of operating the relay. A city deploying hundreds of such nodes therefore gains a monitoring layer without acquiring a single point of failure.

The node was assembled for approximately ₹970 (about US$11–12) and validated against seven fault-injection tests and 8,171 logged telemetry records. Testing established both what the system does and what it cannot see: loss of pump power goes undetected while the firmware continues to report that irrigation is in progress. The project's unique outcome, set out in Section 6.3, is a measurement of how much conventional evaluation inflates soil-image classification accuracy — 45.0 to 47.9 percentage points on this dataset.

# 1. Introduction

## 1.1 Purpose

This document specifies the requirements, design, implementation, testing and outcome of an edge-based irrigation controller for urban green infrastructure. It is intended to be read by the project supervisor and evaluators, and to be sufficient for another engineer to rebuild and re-evaluate the system from the information it contains.

## 1.2 Scope

The system covers measurement of soil and ambient conditions, autonomous actuation of an irrigation pump, telemetry capture, soil classification from photographs, weather-aware watering advice and a monitoring interface. It does not cover multi-zone valve control, wireless backhaul, enclosure design or volumetric metering of water delivered; these are identified as future scope in Section 6.6.

## 1.3 Smart-city context

Water is one of the core utility domains of a smart city, alongside energy, mobility and waste. Within that domain, irrigation of public green space is unusual in two respects: it is a large and continuous consumer of treated or recycled water, and it is almost always controlled open-loop, by schedule rather than by measurement.

Green infrastructure is simultaneously becoming more important to urban planning, because vegetation moderates the urban heat-island effect, manages stormwater runoff and improves air quality. Cities are therefore being asked to maintain more green space, more reliably, without increasing water consumption. Demand-driven irrigation is the direct response to that pressure.

A second requirement comes from the nature of city-scale deployment. A system that works as a single demonstration unit may fail as a fleet of four hundred. Three properties decide whether it scales: unit cost, tolerance of intermittent connectivity, and whether a failed unit announces itself or fails silently. All three are treated as first-class requirements in Section 2 rather than as afterthoughts.

## 1.4 Definitions, acronyms and abbreviations

| Term | Meaning |
| --- | --- |
| ADC | Analogue-to-digital converter; the ESP32's is 12-bit (0–4095) |
| Dead band | The moisture range between the two thresholds in which the pump holds its existing state |
| Edge computing | Processing and decision-making performed on the device itself rather than on a remote server |
| GPIO | General-purpose input/output pin |
| Hysteresis | Control using two thresholds rather than one, so that the switching point depends on the current state |
| Open-loop control | Control in which the result of an action is never measured or fed back |
| SDS | Software Design Specification |
| Source-disjoint evaluation | Evaluation in which every sample derived from a held-out photograph is excluded from training |
| SRS | Software Requirements Specification |
| SVM | Support vector machine |
| Telemetry | Measurement data transmitted from the node for monitoring |

Table 1.1: Definitions and abbreviations.

## 1.5 Overview of this document

Section 2 is the Software Requirements Specification: what the system must do and what it must survive. Section 3 is the Software Design Specification: architecture, hardware, data and module design, with the rationale for each major decision. Section 4 describes the implementation, Section 5 the testing, and Section 6 the outcome, including a requirements traceability matrix and the project's unique outcome.

# 2. Software Requirements Specification (SRS)

## 2.1 Product perspective

The system is a self-contained irrigation node intended to be replicated across heterogeneous urban green space, reporting upward to a municipal monitoring layer. It replaces a timer-based irrigation controller. It is not a component of an existing product line and has no external system dependencies at run time other than an optional weather API used for advisory purposes only.

## 2.2 Product functions

Measure soil moisture and ambient temperature and humidity.

Decide autonomously when to irrigate, and actuate a pump accordingly.

Report measurements and actuation state continuously.

Classify soil type and condition from a photograph.

Produce a watering advisory from soil state and a weather forecast.

Present live and historical data through a monitoring interface.

## 2.3 User characteristics and stakeholders

| Stakeholder | Characteristics | Need addressed |
| --- | --- | --- |
| Municipal parks and horticulture department | Non-specialist operators; manage many dispersed sites | Reduce water consumption; know which sites are actually being watered |
| City water utility | Analytical; works from consumption data | Lower discretionary demand; per-site consumption data |
| Smart-city operations centre | Monitors distributed assets in real time | Live asset status that does not depend on a perfect network |
| Maintenance crews | Field staff, often without instruments | Be directed to sites needing attention; read node state on sight |
| Residents and community gardeners | Non-technical, cost-sensitive | Affordable automation for terrace gardens and community plots |

Table 2.1: Stakeholders and their characteristics.

## 2.4 Functional requirements

| ID | Requirement | Priority |
| --- | --- | --- |
| FR-1 | The system shall measure volumetric soil moisture and report it as a percentage in the range 0–100 %. | High |
| FR-2 | The system shall measure ambient air temperature and relative humidity. | Medium |
| FR-3 | The system shall switch an irrigation pump automatically based on measured soil moisture, without human intervention. | High |
| FR-4 | The control law shall use two thresholds with a dead band, so that sensor noise near a single set point cannot cause repeated relay switching. | High |
| FR-5 | The system shall transmit a structured telemetry record at a fixed interval of one second. | High |
| FR-6 | The system shall indicate its operating state locally, visible to a maintenance crew without instrumentation. | Medium |
| FR-7 | The system shall classify soil type and soil condition from a photograph. | Medium |
| FR-8 | The system shall generate a watering advisory combining soil state with a 48-hour weather forecast. | Medium |
| FR-9 | The system shall provide a monitoring interface showing live values, history and model output, with report export. | Medium |
| FR-10 | The system shall permit a supervisory manual override of the pump. | Low |

Table 2.2: Functional requirements.

## 2.5 Non-functional requirements

| ID | Requirement | Rationale at city scale |
| --- | --- | --- |
| NFR-1 | The irrigation decision shall be taken on the node itself and shall not depend on any network or remote service. | A fleet cannot assume reliable connectivity; network loss must not stop irrigation |
| NFR-2 | Unit cost shall be low enough for deployment in quantity. | Cost per node, not total system cost, determines feasible coverage |
| NFR-3 | Failure of a secondary sensor shall not stop the primary irrigation function. | Partial degradation is preferable to total outage across a fleet |
| NFR-4 | The monitoring layer shall be incapable of actuating the pump. | Removes the remote attack surface and the single point of failure |
| NFR-5 | The system shall be assembled from commodity components available from ordinary retailers. | Procurement, repair and replacement at municipal scale |
| NFR-6 | Undetectable failure modes shall be identified and documented. | A dashboard that misreports asset state erodes operator trust |

Table 2.3: Non-functional requirements.

## 2.6 External interface requirements

### 2.6.1 Hardware interfaces

The node interfaces with a DHT11 over a single-wire digital protocol on GPIO 4, with a capacitive moisture probe through a 12-bit analogue input on GPIO 34 (ADC1_CH6), and with a 5 V relay module through a BC547B transistor driven from GPIO 26. Two LEDs on GPIO 13 and GPIO 14 provide local state indication. Detailed pin assignments appear in Section 3.4.

### 2.6.2 Software interfaces

The node emits newline-delimited JSON over USB serial at 115200 baud. The host collector consumes that stream and writes a CSV telemetry log. The advisory module consumes a JSON forecast from the Open-Meteo HTTP API. Schemas for both are given in Section 3.5.

### 2.6.3 User interfaces

Two interfaces exist. On the node, a red LED indicates that the system is powered and running and a green LED blinks at 250 ms while the pump is active — readable by field staff without any instrument. On the host, a browser dashboard presents live values, history, classification results, the watering advisory and PDF export.

### 2.6.4 Communication interfaces

The node-to-host link is wired USB serial. No wireless interface is implemented in this phase. The advisory module requires outbound HTTPS to the forecast API; if that request fails, the advisory is omitted and all other functions, including irrigation, continue unaffected.

## 2.7 Constraints, assumptions and dependencies

GPIO 34 is input-only and not 5 V tolerant, which constrains the probe to 3.3 V operation.

The probe calibration constants are valid only at the supply voltage at which they were measured.

The prototype is validated in a controlled soil container, not a live municipal installation.

One probe represents the moisture of its irrigation zone; spatial variation within a zone is not captured.

Mains power and a water source are assumed available at the site, as they generally are at managed urban green space.

Volumetric water saving is not measured, because the prototype has no flow meter. Claims are therefore restricted to what was measured: actuation behaviour and control-law performance.

# 3. Software Design Specification (SDS)

## 3.1 Architectural design

The system is designed in four layers. The boundary between the first layer and the rest is deliberate and is the central design decision of the project: everything required to keep vegetation watered lives on the microcontroller, and every layer above it is additive.

Sensing and control layer — the ESP32 reads both sensors, applies the control law, drives the relay and status LEDs, and emits one JSON record per second.

Communication layer — a serial link at 115200 baud carries that stream to the host.

Processing layer — a collector parses and stores the telemetry; the soil classifiers and advisory logic run here.

Presentation layer — a browser dashboard provides live values, history, model output, watering advice and report export.

Figure 3.1: Layered system architecture. The control loop closes on the ESP32; everything beyond the serial link is monitoring and advisory and cannot actuate the relay.

## 3.2 Deployment design

At city scale the same node is replicated across heterogeneous green spaces. Each instance closes its own loop locally and reports upward. The municipal dashboard aggregates fleet status, consumption trends and maintenance alerts, but the direction of authority is strictly one-way: telemetry flows up, commands do not flow down. This satisfies NFR-1 and NFR-4 simultaneously, and changes the failure characteristics of the fleet — a network outage degrades visibility, not service.

Figure 3.2: Urban deployment design. Each site closes its irrigation loop locally; the city layer monitors and plans but cannot actuate.

## 3.3 Hardware design and component selection

| Component | Specification | Role |
| --- | --- | --- |
| ESP32-WROOM-32 development board | Dual-core 32-bit MCU, 240 MHz, 12-bit ADC (0–4095), 3.3 V logic | Executes the control loop, drives the relay and LEDs, emits telemetry |
| Capacitive soil moisture sensor v1.2 | Analogue output; capacitive sensing avoids the exposed electrodes that corrode in resistive probes; operated at 3.3 V | Primary measurement driving the irrigation decision |
| DHT11 temperature and humidity module | 0–50 °C (±2 °C), 20–90 % RH (±5 %), single-wire digital | Ambient conditions for logging and evaporation-aware advice |
| 5 V single-channel relay module | SPDT contacts, opto-isolated input, 5 V coil | Isolates control electronics from the pump circuit |
| BC547B NPN transistor with 1 kΩ base resistor | TO-92 general-purpose NPN | Relay driver; the GPIO cannot source the relay coil current |
| DC mini submersible pump | 3–6 V DC | Delivers water when the relay closes |
| Status LEDs with 220 Ω resistors | Standard 5 mm LEDs | Local state indication for maintenance crews (FR-6) |
| 5 V external supply | USB power bank or mains adaptor | Powers relay coil and pump on a separate rail with common ground |

Table 3.1: Hardware components and their roles.

Unit cost is a design parameter, not an outcome (NFR-2). Four purchased modules account for ₹470; the balance covers the pump, transistor, LEDs, resistors, breadboard, wiring and tubing.

| Item | Qty | Price (₹) |
| --- | --- | --- |
| ESP32 development board (ESP-WROOM-32) | 1 | 284 |
| Capacitive soil moisture sensor v1.2 | 1 | 83 |
| DHT11 temperature and humidity module | 1 | 61 |
| 5 V single-channel relay module | 1 | 42 |
| Subtotal, itemised modules |  | 470 |
| Pump, BC547B, LEDs, resistors, breadboard, wiring, tubing | — | 500 |
| Total per node |  | 970 |

Table 3.2: Bill of materials per node.

At ₹970 per node, instrumenting 100 irrigation zones costs approximately ₹97,000 in hardware and 400 zones approximately ₹3.9 lakh. These figures are arithmetic on the unit cost and exclude installation, enclosures, plumbing and maintenance; they indicate the order of magnitude at which this becomes a procurement question rather than a research one.

## 3.4 Circuit design

Both sensors are powered from the ESP32's 3.3 V rail. The DHT11 data line is pulled up to 3.3 V through 10 kΩ, and the probe's analogue output is read on GPIO 34. The relay is driven indirectly: GPIO 26 feeds the BC547B base through 1 kΩ, and the transistor switches the relay module input. The pump runs from a separate 5 V supply sharing a ground with the ESP32, so the relay contact switches only the pump circuit.

Figure 3.3: Circuit schematic showing the sensors on the 3.3 V rail, the BC547B relay driver, the status LEDs and the separately supplied pump.

| ESP32 pin | Connected to | Through | Function |
| --- | --- | --- | --- |
| GPIO 4 | DHT11 data line | 10 kΩ pull-up to 3.3 V | Air temperature and humidity |
| GPIO 34 | Moisture sensor analogue output | Direct (ADC1_CH6) | Soil moisture |
| GPIO 26 | BC547B base | 1 kΩ | Relay drive |
| GPIO 13 | Red LED anode | 220 Ω | System alive indicator |
| GPIO 14 | Green LED anode | 220 Ω | Pump running indicator |
| 3.3 V | DHT11 and moisture sensor supply | — | Sensor power |
| GND | Common ground rail | — | Shared with the pump supply |

Table 3.3: Pin assignments.

## 3.5 Data design

Three data structures carry information through the system.

Telemetry record emitted by the node, once per second:

| Field | Type | Range / units | Notes |
| --- | --- | --- | --- |
| temperature | float or null | 0–50 °C | null if the DHT11 read fails |
| humidity | float or null | 20–90 % RH | null if the DHT11 read fails |
| moisture | integer | 0–100 % | after mapping and clamping |
| soil_raw | integer | 0–4095 | raw 12-bit ADC value |
| pump | string | “ON” or “OFF” | commanded state, not measured |

Table 3.4: Telemetry record schema.

Persisted telemetry log (CSV), one row per received record:

| Column | Meaning |
| --- | --- |
| timestamp | Host-side receive time |
| soil_temp, soil_humidity, soil_moisture | Values from the node |
| status, action | Derived irrigation state and action taken |
| outside_temp, wind_speed, city | Context from the weather API |

Table 3.5: Telemetry log schema.

Feature vector used by the soil-type classifier:

| Component | Dimensions | Description |
| --- | --- | --- |
| CNN embedding | 1,280 | Frozen ImageNet backbone, global average pooling |
| Colour histogram | 96 | 32 bins per RGB channel, each normalised to sum to 1 |
| Channel statistics | 6 | Mean and standard deviation per channel, divided by 255 |
| Total | 1,382 | Input to the support vector machine |

Table 3.6: Soil-type feature vector design.

## 3.6 Component and module design

| Module | Responsibility | Key interface |
| --- | --- | --- |
| smart_agri.ino | Sensing, control law, actuation, telemetry emission | USB serial, newline-delimited JSON |
| edge/serial_collector.py | Parse the stream, persist to CSV and status file | Reads serial; writes data/latest.json and sensor_log.csv |
| app/soil_predictor.py | Soil type inference | Image path or object in; class and probabilities out |
| app/soil_condition_predictor.py | Soil condition inference | Image in; condition, confidence and advice out |
| app/irrigation_scheduler.py | Forecast retrieval and watering advisory | Open-Meteo HTTP API; returns schedule windows |
| planner/crop_planner.py | Crop suitability reasoning and reporting | Soil and weather in; suitability report out |
| app/dashboard.py | Monitoring interface and orchestration | Streamlit on localhost:8501 |
| training/source_disjoint_eval.py | Dual-protocol classifier evaluation | Writes results JSON consumed by the figure scripts |

Table 3.7: Module responsibilities.

## 3.7 Control-law design

The probe is calibrated at two reference points measured on the assembled hardware: the raw ADC reading in dry air, A = 2590, maps to 0 % moisture, and the reading in water, W = 1500, maps to 100 %. Intermediate readings are interpolated and clamped:

M = clamp( 100 × (r − A) / (W − A), 0, 100 )

The clamp is a safety feature as much as a convenience: when the probe was deliberately disconnected during testing, the floating input read full scale and the clamp returned 0 % rather than a large negative value.

The actuation rule uses two thresholds rather than one. A single set point makes the relay chatter whenever readings hover near it, because real measurements fluctuate by a point or two. Here the pump turns on below 30 % moisture, turns off above 60 %, and between those values holds its existing state. The 30-point dead band is wider than the observed sensor noise, so noise alone cannot toggle the relay. For a municipal fleet this directly extends relay and pump service life.

## 3.8 Machine-learning pipeline design

Soil images are resized to 224 × 224 pixels and passed through a convolutional neural network pre-trained on ImageNet with its weights frozen. Training a network from scratch would require far more images than this project can collect, so the network is used as a feature extractor and a support vector machine is trained on its output. Classifier hyperparameters were fixed before any evaluation and never tuned, and the feature scaler is fitted inside each training fold rather than on the full dataset, so that neither can leak information from the test data.

## 3.9 Design decisions and rationale

| Decision | Alternative rejected | Reason |
| --- | --- | --- |
| Control loop on the microcontroller | Cloud or gateway decision-making | A fleet cannot assume connectivity; network loss must not stop irrigation (NFR-1) |
| Two-threshold hysteresis | Single set point | Prevents relay chatter from sensor noise; extends actuator life (FR-4) |
| Monitoring layer has no actuation path | Bidirectional cloud control | Eliminates a remote failure and attack surface (NFR-4) |
| Capacitive probe | Resistive probe | No exposed electrodes, so far less corrosion in continuously moist soil |
| Transistor-driven relay module | Direct GPIO drive | The GPIO cannot source the relay coil current |
| Transfer learning with a frozen backbone | Training a CNN from scratch | Appropriate to a small image dataset |
| Separate pump supply with common ground | Single shared supply | Keeps motor switching transients off the logic rail |

Table 3.8: Design decisions and their rationale.

# 4. Implementation

## 4.1 Firmware

The firmware is written in C/C++ for the Arduino core. After initialisation — configuring GPIO directions, starting the DHT11 driver, forcing the relay off, turning the red LED on and allowing the sensor two seconds to stabilise — it repeats the following cycle once per second:

Read the DHT11 on GPIO 4. If the read fails, temperature and humidity are transmitted as null and the cycle continues; irrigation is never blocked by a failed air sensor (NFR-3).

Read the capacitive probe on GPIO 34 as a raw 12-bit ADC value.

Convert that value to a moisture percentage and clamp it to 0–100 %.

Apply the hysteresis rule to determine the pump state.

Update the LEDs — red steady for alive, green blinking at 250 ms while the pump runs.

Emit one JSON record and delay one second.

## 4.2 Calibration procedure

Calibration is performed once per probe and is the step most likely to be done wrong in a field deployment, so it is documented explicitly. The probe is held in dry air and the raw ADC value recorded; it is then immersed in water to its marked depth and the value recorded again. These two constants are written into the firmware. Because both are measured at the probe's operating voltage, the probe must be powered at 3.3 V in service, as it was during calibration.

## 4.3 Host telemetry collector

A Python collector built on pyserial opens the serial port, parses each JSON line, writes the most recent reading to a status file and appends every record to a CSV log. The log is the basis for the control-law analysis in Section 5.3 and would, in a municipal deployment, be the feed into the city data platform.

## 4.4 Classifier pipeline

The training pipeline loads the source photographs, expands each into many samples through cropping, rotation and augmentation, extracts features with the frozen backbone, concatenates the colour descriptor where applicable, standardises the features and fits the support vector machine.

## 4.5 Dashboard and advisory

The dashboard is a Streamlit application served locally. It presents live sensor values as gauges, plots the logged history, accepts a soil photograph for classification, and combines the classified soil condition with a 48-hour Open-Meteo forecast to produce a watering advisory: whether irrigation is needed now, expected rainfall over the next 6, 24 and 48 hours, and which upcoming hours are best for watering given temperature and humidity. A suitability report can be exported as PDF. Supervisory manual pump control is provided (FR-10) as a dashboard function operating within the firmware's own state machine, not as a remote actuation path.

## 4.6 Prototype assembly

The node was assembled on a breadboard with the probe inserted into a sealed soil container that could be wetted and allowed to dry, giving repeatable control over the variable under test.

Figure 4.1: The assembled prototype node, with the ESP32, relay module, sensors, pump and external supply labelled.

## 4.7 Technology stack

| Layer | Technology |
| --- | --- |
| Firmware | C/C++ on the Arduino core; DHT sensor library |
| Telemetry transport | USB serial, 115200 baud, newline-delimited JSON |
| Collector | Python 3.11, pyserial |
| Machine learning | TensorFlow / Keras (frozen backbones), scikit-learn |
| Data handling | NumPy, pandas |
| Interface | Streamlit, Plotly |
| Imaging | Pillow (PIL) |
| Reporting | ReportLab |
| External service | Open-Meteo forecast API |

Table 4.1: Implementation technology stack.

# 5. Testing

## 5.1 Test strategy

Testing was organised around three questions. Does the node behave as specified under normal conditions and at the threshold boundaries? What happens when a component fails? And does the soil classifier perform as well on soil it has never seen as its headline accuracy suggests? The second and third questions matter most for a municipal deployment, because both concern whether the system can be trusted when nobody is watching it.

## 5.2 Functional and fault-injection testing

| Test | Condition applied | Expected | Observed |
| --- | --- | --- | --- |
| TC-01 | Moisture 65 %, 28 °C, 60 % RH | Pump off; red LED on; telemetry sent | As expected — pass |
| TC-02 | Moisture 22 % | Pump on; green LED blinking at 250 ms | As expected — pass |
| TC-03 | Moisture 45 % with pump already on | Pump stays on | State held, no chatter — pass |
| TC-04 | Moisture exactly 30 % | Pump off (rule is M < 30) | As expected — pass |
| TC-05 | DHT11 data line removed during operation | Nulls in telemetry; irrigation continues | As expected — pass |
| TC-06 | Moisture probe unplugged | Reading clamps to 0 %; pump turns on | Pump turned on as a fail-safe, but the fault itself was not detected |
| TC-07 | Pump supply removed while running | Pump stops; ESP32 keeps running | Pump stopped but the firmware kept reporting it on — fail, undetected |

Table 5.1: Functional and fault-injection test results.

Five of seven tests passed as specified. TC-06 is a fail-safe rather than a detection: the node waters because the reading falls to 0 %, but it does not know the probe is missing, and a probe that had drifted instead of being unplugged would produce no symptom at all.

TC-07 is the most consequential result. When the pump supply was cut, the pump stopped, but the firmware held its internal state at “on”, kept the green LED blinking and kept reporting irrigation. Nothing in the system could tell that no water was flowing. In a municipal deployment this failure would persist until vegetation visibly declined, and every record logged in the interval would state that the site was being watered. The cause is structural: the relay output is commanded but never measured.

## 5.3 Control-law validation

The control law was validated against 8,171 telemetry records captured over 33 logging sessions totalling about 4.5 hours, with a median sampling interval of 1.00 s. For each session the pump state was reconstructed from the recorded moisture under two rules: a single set point at 30 %, and the implemented hysteresis rule.

Figure 5.1: A measured 14-minute drying and rewetting cycle, with the pump state reconstructed under both rules. The single set point would have switched the relay six times; the hysteresis rule switched it four times.

Across all sessions the hysteresis rule reduced relay transitions by about 17 % relative to a single set point. The benefit is moderate because only 6.6 % of logged readings fall inside the 30–60 % dead band. In a continuously operating municipal installation, however, a 17 % reduction in actuation count accumulates directly into relay and pump service life.

## 5.4 Classifier evaluation

The image datasets were built by expanding a small number of source photographs into many samples through cropping, rotation and augmentation. This is standard practice with small datasets, but it introduces a hazard: if the expanded samples are shuffled before splitting into training and test sets, near-identical crops of the same photograph land on both sides of the split, and the classifier is partly tested on images it has effectively already seen.

The same classifier and features were therefore evaluated twice:

Augmentation-level evaluation — the conventional approach, shuffling all expanded samples before splitting.

Source-disjoint evaluation — every sample derived from a held-out photograph is excluded from training, so the classifier is tested only on soil it has genuinely never seen.

| Task | Augmentation-level | Source-disjoint | Gap |
| --- | --- | --- | --- |
| Soil type (5 classes) | 91.9–94.0 % | 44.6–46.8 % | 45.0–47.9 points |
| Soil condition (4 classes) | 98.6–99.7 % | 75.5–86.1 % | 13.6–23.1 points |

Table 5.2: Accuracy under the two evaluation protocols.

Figure 5.2: Accuracy under both protocols for three network backbones. The dotted line is the majority-class baseline.

Soil-type accuracy falls to 44.6–46.8 %, close to the 43.3 % obtainable by always guessing the most common class. The result held across three network architectures and five independent randomisations, so it is a property of the dataset rather than of one model. Soil condition holds up considerably better at 75.5–86.1 %, about three times its baseline, and is the more usable of the two classifiers.

## 5.5 Telemetry reliability

Across 7,949 within-session intervals the median time between records was 1.00 s, with an interquartile range of 1.00–2.00 s and a 95th percentile of 3.00 s, bounding how quickly the monitoring layer observes a change. Each fault in Table 5.1 was injected once, so the tests establish behaviour rather than its statistical consistency; repeated injection with instrumented timing is identified as future work.

# 6. Outcome

## 6.1 Requirements traceability

| ID | Status | Evidence |
| --- | --- | --- |
| FR-1 | Met | Moisture reported 0–100 % from a two-point calibration; 8,171 records logged |
| FR-2 | Met | DHT11 temperature and humidity logged throughout |
| FR-3 | Met | TC-01 to TC-04: pump switched automatically from the moisture reading |
| FR-4 | Met | TC-03 and Section 5.3: no chatter in the dead band; 17 % fewer transitions than a single set point |
| FR-5 | Met | One JSON record per second; median interval 1.00 s |
| FR-6 | Met | Red and green LEDs; verified in TC-01 and TC-02 |
| FR-7 | Partially met | Soil condition usable at 75.5–86.1 %; soil type not usable at 44.6–46.8 % (Section 5.4) |
| FR-8 | Met | Advisory generated from soil condition and a 48 h Open-Meteo forecast |
| FR-9 | Met | Streamlit dashboard with live values, history, model output and PDF export |
| FR-10 | Met | Supervisory manual override implemented in the dashboard |
| NFR-1 | Met | Control loop executes entirely on the ESP32; verified with the host disconnected |
| NFR-2 | Met | ₹970 per node, about a quarter of the cloud-dependent baseline |
| NFR-3 | Met | TC-05: irrigation continued on moisture alone after DHT11 failure |
| NFR-4 | Met | By construction — no actuation path exists from the host to the relay |
| NFR-5 | Met | All components sourced from a general electronics retailer |
| NFR-6 | Met | TC-06 and TC-07 documented as undetectable with the present hardware |

Table 6.1: Requirements traceability matrix.

## 6.2 Quantitative outcomes

| Measure | Result |
| --- | --- |
| Cost per node | ₹970 (approx. US$11–12) |
| Cost relative to cloud-dependent baseline | About one quarter |
| Control-loop execution | Entirely on-node, 1 Hz, no network dependence |
| Reduction in relay transitions vs single set point | About 17 % |
| Telemetry captured | 8,171 records, 33 sessions, approx. 4.5 hours |
| Median telemetry interval | 1.00 s (IQR 1.00–2.00 s) |
| Functional tests passed | 5 of 7; 2 documented as undetected faults |
| Soil condition accuracy (unseen photographs) | 75.5–86.1 % |
| Soil type accuracy (unseen photographs) | 44.6–46.8 % — not deployable |
| Requirements met | 15 of 16 fully; 1 partially |

Table 6.2: Summary of measured outcomes.

## 6.3 Unique outcome: measuring evaluation inflation in soil-image classification

Beyond the working node, this project produced a result that is transferable to other smart-city machine-learning deployments and that, to our knowledge, has not previously been quantified for soil photographs.

Soil-image datasets are commonly built by expanding a handful of source photographs into thousands of samples through cropping and augmentation. Published accuracies for this task routinely exceed 90 %. By evaluating the same classifier, on the same features, under both the conventional protocol and a photograph-level split, this project measured how much of that accuracy is real:

Soil type fell from 91.9–94.0 % to 44.6–46.8 % — a gap of 45.0 to 47.9 percentage points, and barely above the 43.3 % majority-class baseline.

Soil condition fell from 98.6–99.7 % to 75.5–86.1 %.

The soil-type gap held across three network architectures and five independent randomisations, confirming it as a property of the data rather than of one model.

The same features identify which photograph a tile came from with 91.5 % accuracy, against a 6.3 % chance rate — direct evidence that the classifier was recognising photographs, not soils.

Classes represented by a single source photograph score an F1 of exactly zero once that photograph is held out, because the class is then absent from training altogether.

The practical significance for smart-city design is this. Municipal systems increasingly justify operational and procurement decisions by citing model accuracy. An accuracy figure quoted without stating how the data were split, and how many genuinely independent samples underlie it, cannot support such a decision. On this dataset the difference between the two reporting conventions was large enough to invert the conclusion: a classifier that appears deployable at 94 % is, measured honestly, no better than guessing the most common class.

This finding is the basis of the research paper submitted alongside this report as the project outcome.

## 6.4 Outcome against the smart-city objective

The project set out to show that demand-driven irrigation of urban green space can be automated at a unit cost and a reliability profile appropriate to city-scale deployment. On the infrastructure side that objective was met: the node waters on measurement rather than on a timer, costs under a thousand rupees, continues to operate with the host and network both absent, degrades gracefully when its secondary sensor fails, and actuates its relay measurably less often than a naive controller.

The outcome also demonstrates a governance property that matters more at fleet scale than at prototype scale. Because the monitoring layer has no actuation path, adding city-wide visibility does not add a city-wide failure mode. An operations centre can lose its network and lose sight of four hundred nodes without any of them stopping work.

## 6.5 Limitations

Undetected actuator failure. A pump that loses power is reported as running (TC-07).

No probe validation. A drifted, fouled or dislodged probe continues to return plausible values, and the controller trusts them (TC-06).

Volumetric water saving not measured. The prototype has no flow meter, so the report claims only what was measured and makes no claim about litres saved.

Soil-type classification is not deployable at its measured accuracy on unseen photographs.

Container validation only, over hours rather than a season.

Single-point sensing: one probe represents a whole irrigation zone.

Wired telemetry; wireless backhaul was not implemented or tested.

## 6.6 Future scope

Add a current sensor on the pump supply, such as an ACS712 at about ₹145, which closes the TC-07 gap directly by showing whether the pump draws current. At roughly 15 % of unit cost this is the single highest-value addition.

Implement a pulse-response probe check: deliver a known volume and verify that the moisture reading rises, catching a fouled or dislodged probe while correctly passing one that is merely miscalibrated.

Install an inline flow meter so that water saving can be quantified in litres and reported per site.

Migrate the telemetry link to Wi-Fi, LoRaWAN or NB-IoT for city-scale backhaul, keeping the control loop local.

Expand to multi-zone valve control so one node can serve several beds.

Collect several hundred field photographs across soil classes to make soil-type classification viable.

Integrate with a municipal data platform and asset-management workflow so that alerts generate maintenance tickets automatically.

Add enclosure design, surge protection and tamper resistance for street-level deployment.

## 6.7 Conclusion

Urban green infrastructure is watered largely by timers, in cities that increasingly cannot afford to waste water. This project specified, built and tested a node that waters by measurement instead, for about ₹970, with its decision loop held locally so that network failure degrades visibility rather than service. Seven fault tests established what it does and does not handle, and the one failure it cannot see — loss of pump power — is documented together with a ₹145 remedy.

The wider lesson concerns how smart-city systems are evaluated. A controller that reports irrigation it is not performing and a classifier that reports accuracy it does not have fail in the same way: both look correct from the outside. Neither failure was discovered by observing the system work; both were found only by testing specifically for them. For infrastructure a city intends to trust without supervision, that kind of adversarial testing is not an optional extra but the part of the work that determines whether the rest can be believed.

# References

[1]  A. Morchid, H. Qjidaa, R. El Alami, S. Mobayen, P. Skruch and B. Bossoufi, “Smart irrigation-based internet of things and cloud computing technologies for sustainable farming,” Scientific Reports, vol. 16, art. 5293, 2026.

[2]  A. A. Abdelmoneim, H. N. Kimaita, C. M. Al Kalaany, B. Derardja, G. Dragonetti and R. Khadra, “IoT sensing for advanced irrigation management: a systematic review of trends, challenges, and future prospects,” Sensors, vol. 25, no. 7, art. 2291, 2025.

[3]  M. Pramanik et al., “Automation of soil moisture sensor-based basin irrigation system,” Smart Agricultural Technology, vol. 2, art. 100032, 2022.

[4]  R. Sreeram, S. A. Krishna, A. S. Kumar, S. Remya and Y. Y. Cho, “Soil moisture monitoring technologies in smart agriculture: a comprehensive review,” Farming System, vol. 4, no. 2, art. 100189, 2026.

[5]  M. U. Tariq, S. M. Saqib, T. Mazhar, M. A. Khan, T. Shahzad and H. Hamam, “Edge-enabled smart agriculture framework: integrating IoT, lightweight deep learning, and agentic AI for context-aware farming,” Results in Engineering, vol. 28, art. 107342, 2025.

[6]  R. K. Banoth and B. V. Ramana Murthy, “Soil image classification using transfer learning approach: MobileNetV2 with CNN,” SN Computer Science, vol. 5, no. 1, art. 199, 2024.

[7]  S. Jalapur and N. Patil, “An integrated deep learning framework for soil type classification,” SN Computer Science, vol. 6, no. 3, art. 251, 2025.

[8]  S. Kapoor and A. Narayanan, “Leakage and the reproducibility crisis in machine-learning-based science,” Patterns, vol. 4, no. 9, art. 100804, 2023.

[9]  M. Tan and Q. V. Le, “EfficientNetV2: smaller models and faster training,” in Proc. 38th Int. Conf. Machine Learning (ICML), PMLR vol. 139, 2021, pp. 10096–10106.

[10]  D. N. K. Pandiri, R. Murugan and T. Goel, “Indian Regions Soil Image Database (IRSID),” IEEE DataPort, 2021. doi:10.21227/2zz3-f173

[11]  Open-Meteo, “Free weather forecast API.” https://open-meteo.com/ (accessed Oct. 2026).